# Time Sequence

## Safety mechanism event tables

How to read these tables Every row is one of two kinds. Verified rows state a value traced directly to a code parameter (shown in monospace) or a measured log. Pending rows are shown in red italics and mean the timing has not been measured or configured anywhere in this workspace — they are not placeholders to be trusted, they are the outstanding test list. Only two timing values are currently verified: the 250 ms command timeout (diff_controller.yaml:25) and the 100 ms EKF sensor staleness window (ekf_config.yaml:4). All other timings below are pending measurement.

# 1) External E-stop

No wired external E-stop path exists in this workspace today. The rows below describe the intended design based on the CAN protocol scaffolding and hardware interface fallback logic present in the code — not a verified or tested sequence.

| Time | Event | Response |
| --- | --- | --- |
| pending — not wired | Operator presses physical E-stop | Intended: drive-enable or power line to ODrive changes state |
| pending — not confirmed in repo | Power/signal isolation | Intended: physical line cut removes ODrive drive-enable |
| pending | Motor driver shutdown | ODrive axis can no longer command torque (interface-inactive fallback: odrive_hardware_interface.cpp:300,302) |
| pending | CAN heartbeat loss (secondary symptom) | Software layer detects heartbeat loss independent of the hardware-level stop |

Gap declared transparently src/rover_nav/launch/LeapOne_Safety_launch.py references joy_safety.py at line 29, but this node is not present in the workspace. The only E-stop-related code is vendored CAN protocol scaffolding (Estop_msg_t, can_simple_messages.hpp:100; ODRIVE_ERROR_ESTOP_REQUESTED, odrive_enums.h:108), never wired to any node. Do not present this table as a validated sequence until the physical path is implemented and tested.

# 2) Watchdog and Heartbeat Monitoring (T-37)

| Time | Event | Response |
| --- | --- | --- |
| t = 0 | Command stream (cmd_vel) stops updating | Command becomes stale relative to last received timestamp |
| t = 250 ms (verified: diff_controller.yaml:25) | Command timeout window elapses | Controller invalidates stale command and forces commanded velocity to zero |
| pending — not measured | CAN / ODrive heartbeat loss detected | Subsystem treated as unresponsive; interface-inactive fallback requests IDLE (odrive_hardware_interface.cpp:300,302) |

Heartbeat period itself has runtime data: a logged CAN session (rover_nav/CSVs/DIagnosis_of_odrives.csv) shows a median controller_status inter-arrival period of about 250 ms — but this is heartbeat period, not detection latency, and no watchdog-triggered stop event was captured in that recording.

# 3) Autonomous Fail-Safe Mechanism (T-14, T-17, T-37)

| Time | Event | Response |
| --- | --- | --- |
| t = 0 | Degraded control-path condition occurs | Stale command input, inactive control interface, or invalid control mode |
| pending — no benchmark exists | Fail-safe condition detected | hardware_interface.py is a diagnostic/status node with configured check_interval = 3.0 s, timeout = 5.0 s — polling thresholds, never benchmarked for detection latency |
| pending | Rover transitions to safe state | Motion halted and/or actuator IDLE requested (odrive_hardware_interface.cpp:325,326) |

Evidence quality note The one fault event captured in rover_nav/CSVs/DIagnosis_of_odrives.csv (t ≈ 1766083692.21) was ODRIVE_ERROR_SPINOUT_DETECTED, not a watchdog-timeout or heartbeat-loss event — it does not count as fail-safe detection-latency evidence.

# 4) Command Validation and Safety Filtering (T-19, T-14)

| Time | Event | Response |
| --- | --- | --- |
| continuous | Velocity command received | Compared against configured velocity/acceleration limits (diff_controller.yaml:34,37,43,46) |
| pending — loop rate not measured | Out-of-range command detected | Command constrained by the differential-drive controller; limiting output enabled (diff_controller.yaml:27) |
| pending (T-19, T-14) | Validation check rate | Tied to control loop execution rate; not separately measured or documented |

# 5) Sensor Consistency Checks (T-15)

| Time | Event | Response |
| --- | --- | --- |
| continuous | Wheel odometry and IMU inputs fused | EKF combines sources to reject single-source drift (ekf_config.yaml:16, 29) |
| t = 100 ms (verified: ekf_config.yaml:4) | Sensor input exceeds staleness window | EKF freshness check flags the stale input for that source |
| continuous | Wheel-slip outliers filtered | Per-side median filtering applied before fusion (rover_nav/scripts/Odom.py) |

Note: EKF nominal update rate is 20 Hz (ekf_config.yaml:3), which is verified/configured rather than measured under load.